# Optimization Frameworks for Hybrid Biomass Power-to-Methanol System: MILP and Reinforcement Learning Approaches

# Muhammad Rafi Afif Indrajaya

Erasmus Mundus Decentralized Smart Energy System (DENSYS)

# Politecnico di Torino

Submitted on July, 18th 2026

# Abstract

# Acknowledgment

I want to ddedicate this thesis for abladlkafjldkjfal flkad ladshflasdhf lasdkhfasd hflksadhflak hflkadhflkas dhflkasdh flkadshflkasdhflhadlsf hsadlf hadslfh lsadkh flskadhf lkasdhf ldkshf lksdahf lkdashf dss fsd fsdf sdf sdf sdfs df sdfaf asf asf asdf asdf as

Sd fasd fsadf

Fasdf asdf asf asd

Ds afda fsad f

D fas fasdf asdf

Dsf asdf adsf asdf

Df asdf asf

A dfasdf as

Sfsdfsdfadfasdfasdfadsfasdfasf

Dsfadsdfasd

Sdafdasf

Sdafasdf

Fsgsfgdsgsdfg

Dsfgfsgfsd

Gsdfgsdfgfsdgf

Sdfgsfdgsdfg

Sdfgsdfgsdf

Gsdfgsdfgsdf

gsdfgsdfgsdfgdsgdsfg

Table of Contents

1	Introduction	1

1.1	Background	1

1.2	Introduction to Company	2

1.3	Objective	3

1.4	Thesis Configuration	4

2	Literature Review	5

2.1	Power-to-X Integration for Methanol Synthesis System	5

2.2	MILP for Complex Polygeneration System	7

2.3	State-of-the-art RL in Optimizing Energy System and Industrial Processes	8

2.4	RL vs MILP for Different Applications	10

3	Methodology	13

3.1	Hybrid Biomass Power-to-Methanol System Model	13

3.2	Two-steps Outerloop & Innerloop Strategy	13

3.3	Level 2 Optimization Architecture (MILP)	13

3.4	Level 3 Optimization Architecture (RL)	13

3.5	Data	13

3.6	Comparison Stages & Scenario Definition	13

4	Results and Discussions	17

4.1	17

5	Conclusions	18

6	References	19

# List of Abbreviations

# Introduction

## Background

The large-scale transformation of industrial energy systems is widely regarded as a critical enabler for achieving global decarbonization targets, particularly in so-called hard-to-abate sectors [1]. Among the available pathways, Power-to-X (PtX) technologies have emerged as a promising solution, enabling the conversion of renewable electricity into chemical energy carriers and synthetic fuels suitable for industrial applications [2]. However, PtX systems are inherently complex and capital-intensive, especially when their operation is coupled with the variability and intermittency of renewable energy source [3]. This complexity introduces significant challenges in both system design and operational planning, thereby necessitating robust optimization strategies that can effectively minimize capital expenditures (CAPEX) and operational expenditures (OPEX).

To address this need, Mixed-Integer Linear Programming (MILP) has become the prevailing industrial standard due to its robustness, transparency, and well-established solution techniques [4]. At the same time, the rapid advancement of artificial intelligence (AI) has motivated the exploration of alternative methodologies, particularly reinforcement learning (RL) [5]. In this context, RL can be understood as an agent-based framework that learns optimal operational strategies through iterative interaction with the system environment, effectively acting as an autonomous plant operator.

Despite its widespread adoption, MILP relies on linear approximations of system behavior, which can limit model fidelity when representing inherently nonlinear processes [6]. In practice, such nonlinearities are often simplified or neglected. This limitation makes RL particularly attractive, as it allows for flexible environment design in which high-fidelity, nonlinear system dynamics can be directly incorporated. Through appropriately defined reward structures, the RL agent can learn operational strategies that account for these complexities without requiring explicit linearization [7].

In summary, this thesis evaluates the techno-economic performance of a reinforcement learning–based approach against a conventional MILP-based framework for a hybrid biomass power-to-methanol plant located in South Africa. The analysis is performed across multiple scenarios and case studies to capture a range of operating conditions and system configurations. Through this comparative assessment, the study seeks to clarify the role and current capabilities of RL in optimizing complex Power-to-X systems, while benchmarking its performance against an established industrial standard.

## Introduction to Company

This master’s thesis is conducted in cooperation with FEV Europe GmbH, specifically within its energy+resources department. In this industrial context, the research aligns with ongoing efforts to expand capabilities beyond traditional automotive engineering toward energy-related applications.

FEV Europe GmbH is widely recognized as an established expert within the automotive industry, particularly for its benchmarking and consulting services. Building on this foundation, the company is increasingly directing its efforts toward supporting innovation within the energy sector, reflecting broader industry trends and internal strategic priorities.

Within this framework, the present thesis contributes to the identification and evaluation of an efficient optimization tool for innovative energy solutions, with a focus on complex polygeneration plants. The outcomes are intended to support future FEV Europe GmbH projects by enabling improved system design and operational decision-making.

## Objective

This thesis aims to systematically evaluate and compare two optimization frameworks across multiple operational scenarios. The analysis focuses on assessing their ability to represent and optimize complex hybrid energy systems, with particular attention to differences in system configuration and performance outcomes.

Building on this motivation, this thesis adopts a two-step methodological framework to systematically compare different levels of modeling fidelity and optimization approaches. In the first step, a comparison is conducted between a fully integrated MILP formulation and a decoupled framework in which system design (sizing) and operation (dispatch) are optimized separately. This step aims to assess the impact of incorporating nonlinear CAPEX assumptions in the outer-loop (design optimization). In the second step, the resulting optimal system sizing is used as an input for operational optimization, where both MILP and RL-based approaches are applied in the inner-loop (dispatch optimization). This structure enables a direct comparison of the two frameworks in terms of their ability to determine cost-optimal operational strategies, particularly when contrasting linearized system representations with higher-fidelity nonlinear models.

A central objective is to investigate whether a non-linear physical model integrated with a reinforcement learning (RL) agent converges to substantially different component sizing and system layouts compared to a mixed-integer linear programming (MILP) approach. These differences are examined in terms of key techno-economic indicators, including capital expenditure (CAPEX), operational expenditure (OPEX), clipped energy, simulation time, amount of methanol produced and levelized cost of methanol (LCOM).

Based on the simulation results, the thesis further aims to identify and explain the underlying reasons for any observed discrepancies—or the lack thereof—between the two frameworks. This comparative analysis supports a final assessment of whether one framework demonstrates superior performance or whether its suitability is dependent on specific system configurations and operating conditions.

The expected contribution of this work is to enable the selection of an appropriate optimization framework for hybrid methanol synthesis plants, while also providing a structured approach for framework selection in other industrial energy systems. Additionally, the findings aim to improve optimization accuracy and/or reduce computational effort, and to enhance understanding of where and how RL-based approaches can be effectively applied within polygeneration system design.

## Thesis Configuration

This thesis is structured to provide a systematic and transparent comparison of the selected optimization frameworks. Chapter 2 reviews the state-of-the-art literature on the application of reinforcement learning (RL) in optimizing complex industrial processes and energy systems, including existing comparative studies between optimization approaches.

Chapter 3 details the methodological framework adopted for the comparison. It outlines a multi-stage evaluation procedure, describing each step taken to ensure a consistent and rigorous assessment of the two optimization approaches.

Chapter 4 presents and discusses the results of this multi-stage comparison. It analyzes the performance of each framework across different scenarios and case studies, and introduces a generalization scoring approach to assess their robustness and applicability across multiple evaluation dimensions.

Finally, Chapter 5 summarizes the main findings of the thesis and reflects on key limitations, assumptions, and methodological considerations that underpin the analysis, providing context for the interpretation and transferability of the results.

# Literature Review

## Hybrid Biomass Power-to-Methanol System

The hybridization strategy considered in this thesis integrates two upstream pathways that jointly supply the feedstock for methanol synthesis: (i) a biomass-derived pathway and (ii) a hydrogen pathway based on water electrolysis. This configuration enables coordinated carbon and hydrogen management prior to the synthesis loop. In the literature, gasification is most commonly employed to directly convert solid biomass into syngas. An alternative route relies on anaerobic digestion (AD), where biomass is first converted into biogas and then reformed to syngas prior to methanol synthesis. The differences in process steps primarily drive the trade-offs between these technologies. The AD-based route is attractive because it can be integrated with existing waste-treatment infrastructure; however, it requires additional downstream conversion steps, making the pathway less direct and more dependent on reforming, CO₂ handling, and potentially electrolysis integration. In contrast, gasifier-based routes are more established for methanol synthesis, as they provide syngas directly and are well suited for coupling with high-temperature processes. Their disadvantages include challenges related to high-temperature solid handling, the requirement for an oxygen supply (e.g., via an air separation unit, ASU), and stricter gas-cleaning requirements [8].

Table 1 Structural and Process Comparison between AD and Gasification routes

| Metric | Anaerobic Digestion (AD) Route | Biomass Gasification Route | References |
| --- | --- | --- | --- |
| Primary Conversion Mechanism | Biochemical (mesophilic/thermophilic digestion of wet organic feedstocks) | Thermochemical high-temperature reactions (>800∘C-1200∘C) | [9] |
| Intermediate Stream | Biogas (∼55--65% CH4, 35--45% CO2) requiring reforming to form syngas. | Raw syngas (CO,H2,CO2,CH4) containing tars and particulates. | [9], [10] |
| Feedstock Flexibility | High-moisture, biodegradable substrates (e.g., sewage sludge, organic municipal solid waste, agricultural slurry). | Low-moisture solid biomass (e.g., forestry residues, lignocellulosic crop waste, refuse-derived fuel/RDF). | [9] |
| Core Process Bottleneck | Biogas reforming (Dry Reforming DRM / Steam Reforming SMR) and stoichiometric H2/CO ratio adjustment. | Severe syngas conditioning requirements (tar removal, particulate scrubbing, sulfur/halogen cleanup, and Water-Gas Shift). | [9], [11] |
| Techno-Economic Profile | Lower CAPEX per unit capacity; economically viable at smaller, regional plant sizes. | High upfront CAPEX; strongly dependent on economies of scale and centralized feedstock supply chains. | [10] |
| Lifecycle GHG Abatement | Moderate-to-high carbon footprint reduction depending on feedstock cultivation and digestate handling. | Exceptional carbon abatement potential (up to -0.88 t CO2e/t CH3OH) when combined with biogenic carbon capture. | [10] |

The hydrogen pathway begins with the electrochemical splitting of water into hydrogen and oxygen using an electrolyzer.

2H2O→2H2+O2

Equation 1 Electrolysis.

Several studies identify Solid Oxide Electrolyzer Cells (SOECs) as a promising option for power-to-methanol systems. The high operating temperature of SOECs allows for heat integration and potential oxygen utilization, which can increase overall plant efficiency compared to low-temperature systems[12]. However, key challenges include long-term stack durability, technological maturity, and limited operational flexibility, which is particularly relevant for systems coupled to variable renewable electricity. Techno-economic comparisons suggest that alkaline water electrolysis (AWE) represents a viable short- to mid-term option, whereas SOECs are more likely to be competitive in the mid- to long-term. Low-temperature electrolyzers such as AWE and proton exchange membrane (PEM) systems offer higher technological maturity and superior load-following capability, which is advantageous for renewable integration, but they do not fully exploit heat integration or co-electrolysis opportunities available to SOECs [13]. A detailed quantitative synthesis of these performance metrics and operating limits is summarized in Table 2.

Table 2 Parameter comparison between AWE, PEM, and SOEC [14], [15], [16].

| Parameter | Alkaline Water Electrolysis (AWE) | Proton Exchange Membrane (PEM) | Solid Oxide Electrolysis (SOEC) |
| --- | --- | --- | --- |
| Operating Temperature (​∘C) | 60-90∘C | 50-80∘C | 650-1000∘C |
| Operating Pressure (bar) | <30 bar | 30-80 bar | <30 bar |
| Energy Consumption (kWh/Nm3 H2) | 4.5-5.5 | 4.2-5.5 | 3.2-3.8 |
| Minimum Operational Load (%Pnom) | 10%-20% | 0%-10% | 20%-30% |
| Dynamic Response | Moderate (0.2--20%/s) | Very High (10--100%/s) | Low (<1%/s) |
| H2 Purity (%) | >99.5% | >99.999% | >99.9% |
| Commercial Status / TRL | Commercial / State-of-the-art (TRL 9) | Near-commercial (TRL 8-9) | R&D (TRL 6-7) |

In hybrid biomass and power-to-methanol (PBtM) systems, the technical consensus across recent literature emphasizes the need to achieve an optimal blending between the carbon-rich syngas with the pure hydrogen coming from the electrolyzer to achieve complete carbon efficiency [17]. Three repeating strategies are used in the literature for this specific goal:

- Direct hydrogen blending upstream of methanol loop compressor:

Biogas goes through a reformer to produce a primary syngas containing CO, CO2, and H2. Due to the low stoichiometric module below the required threshold of around 1.5 – 1.8, the blending with hydrogen before the methanol reactor increases this number to >2, enabling 100% biogenic carbon recovery without venting CO2 [18].

- Reverse Water-Gas Shift (RWGS) Coupling Junction:

Unrefined or partially reformed biogas CO2 is routed into an RWGS reactor

CO2+H2⇌CO+H2O

Equation 2 RWGS chemical reaction.

Where it explicitly mixes with a portion of the green hydrogen stream at 600-800∘C. Direct CO2 hydrogenation to methanol produces one mole of water for every mole of methanol,

CO2+3H2⇌CH3OH+H2O

Equation 3 CO2- based Methanol synthesis.

which accelerates catalyst deactivation through kinetic inhibition and active site sintering [17]. By meeting in an RWGS unit prior to the methanol synthesis, the hydrogen reduces CO2 to CO, shifting the synthesis reaction toward the faster, more thermodynamically favorable CO hydrogenation route [19].

CO+2H2⇌CH3OH

Equation 4 CO-based Methanol synthesis.

- Anodic Oxygen Integration at the Reforming Stage:

In this strategy, the byproduct of the electrolysis step, O2, already meets the biogas path inside an Autothermal Reformer (ATR). The oxygen from the electrolyzer will then be used to generate internal thermal energy via exothermic methane partial oxidation

2CH4+O2+H2O→2CO+5H2

Equation 5 ATR checmical reaction.

Which sustains the endothermic steam reforming reactions. This strategy eliminates the need for a capital-intensive Air Separation Unit (ASU). Dossow et al. [20] demonstrated that full oxygen integration between water electrolysis and thermochemical reforming reduces overall auxiliary plant electrical consumption by 8-12%, establishing a closed mass-and-energy synergy between the hydrogen and biogas paths.

While these strategies establish the thermodynamic and chemical validity of stream integration, maintaining mass-balance ratios between the continuous biogas path and volatile power-dependent hydrogen path remains a central operational dispatch challenge requiring advanced optimization strategies [21].

The hybrid approach to methanol production is attractive due to its potential for higher carbon utilization and improved use of biomass resources. One study reports a carbon utilization of 89.8% and an overall efficiency of 85.8%, with up to 101% higher methanol production per unit of biomass compared to a conventional biomass-to-methanol baseline [22]. Another analysis indicates that a state-of-the-art biomass-to-methanol process achieves approximately 53.3% efficiency with a 4.8-year payback period, while integrating electrolysis for full carbon conversion increases efficiency to 64.5–66.0%, albeit with less favorable economics [23]. Furthermore, under favorable electricity price conditions, flexible operation can allow the plant to capture value from temporal price variations instead of operating continuously at a fixed load [24].

Despite these advantages, several bottlenecks challenge the feasibility of hybrid methanol systems. These include electrolyzer utilization rates, electricity price volatility, capital cost and lifetime of electrolyzer stacks, as well as the integration of oxygen and heat streams. For example, it has been reported that high electrolyzer capacity factors are required for competitive e-methanol production, and that full-carbon-conversion configurations become attractive only when improvements in electricity costs, stack lifetime, and capital expenditure are realized [25]. Oxygen management represents an additional practical constraint: while direct integration of oxygen streams may reduce the need for large-scale oxygen storage and improve capital efficiency and safety, this is contingent on matching oxygen supply and demand within the plant [21].

To address these challenges, process and system-level optimization emerges as a critical tool for improving both technical and economic performance. For instance, optimization of the methanol synthesis reactor in a power-and-biomass-to-methanol (PBtM) configuration has been shown to reduce e-methanol production costs by approximately 20% [25]. While such studies typically focus on individual subsystems, it is reasonable to expect that plant-wide optimization could yield further cost reductions and efficiency gains. In this context, optimization acts as the bridge between technically feasible system designs and configurations that remain viable under realistic operating conditions, including fluctuating electricity prices, electrolyzer degradation, and large-scale capital constraints.

## MILP for Complex Polygeneration System

MILP, or Mixed-Integer Linear Programming, has been used as a tool to optimize different kinds of energy or industrial plants by linearizing the model of these processes and setting up a linear objective function to either maximize or minimize specific key performance indices (KPIs) based on the optimization objective [26]. In an energy system that includes the flow of electricity, heat, and mass, the constraints considered will then dictate how the balance of the system should behave, the minimum and maximum operational dispatch, and the evolution of the storage state of charge, if storage exists. On the other hand, the objective function mainly includes the capital expenditure (CAPEX) and operational expenditure (OPEX) of the models defined in the system.

minZ=c∈CCAPEXc⋅r1+rnc1+rnc-1+OPEXfix,c+t=1TCelectricityt+Cfeedstockt+Cvart⋅Δt

Equation 6 Objective function example of an energy system

g∈GPgt+Pgrid, buyt+Pbat, dist=Pdemandt+Pgrid, sellt+Pbat, cht+c∈CPct ∀t∈T

Equation 7 Power balance constraint example

Pmin,c⋅yct≤Pct≤Pmax,c⋅yct, yct∈0,1 ∀c∈C,∀t∈T

Equation 8 Example of a flow constraint

SoCt=SoCt-1⋅1-σ+ηch⋅Pbat, cht-Pbat, distηdis⋅Δt ∀t∈T

Equation 9 Example of SOC evolution definition in MILP.

Together, the MILP will generate a global equation that accounts for the constraints and the objective function, and, using the MILP solver, it will perform relaxation and branch-and-bound to find the valleys or hills within the linear boundaries. In practice, MILP has been used as the reference method for operational optimization, demand response, scheduling in energy communities, industrial plants, and integrated multi-energy systems because it produces globally optimal solutions to the defined linear problem when sufficient computation time is available [27].

Selecting an appropriate optimization methodology for complex polygeneration systems requires balancing physical model accuracy, computational tractability, and handling of operational uncertainty [28]. Adjacent to Mixed-Integer Linear Programming (MILP), several mathematical and heuristic paradigms are widely evaluated across the literature [29]. Mixed-Integer Non-Linear Programming (MINLP) is employed when non-linear thermo-chemical physics are prevalent; however, as the system scale expands, non-convexities can cause computational complexity to grow exponentially, rendering large-scale models intractable or prone to local sub-optima [30]. Stochastic programming explicitly addresses parameter uncertainty by evaluating system states across multi-scenario trees, but its computational burden rises sharply with increasing scenario counts and temporal horizons [31]. For non-linear, multi-objective hybrid energy problems, metaheuristics such as Particle Swarm Optimization (PSO) and Genetic Algorithms (GA) are frequently adopted due to their model-free flexibility and robustness; yet, unlike exact MILP solvers, they offer no mathematical guarantee of global optimality [32]. To bridge these trade-offs, hierarchical and bilevel optimization architectures decouple strategic capacity sizing (e.g., using metaheuristics or Bayesian optimization) from high-frequency operational dispatch (e.g., solved via MILP or Reinforcement Learning), a strategy detailed in subsequent sub-sections [33].

Table 3 Advantages and Disadvantages of different optimization paradigms [32], [33], [34], [35].

| Optimization Paradigm | Key Advantages | Key Disadvantages |
| --- | --- | --- |
| MILP (Mixed-Integer Linear Programming) | • Proven global optimality. • Ultra-fast commercial solvers (Gurobi/CPLEX). • High bankability & industrial standard. | • Cannot natively handle non-linear physics or exponential cost curves without piecewise linear approximations. |
| MINLP (Mixed-Integer Non-Linear Programming) | • Captures highly realistic physical/chemical behavior without linear simplifications. | • Non-convex problems can get trapped in local sub-optima. • High computational complexity (NP-hard). |
| Stochastic Programming | • Explicitly accounts for parameter uncertainty (e.g., volatile weather, price shocks). | • Suffers from scenario explosion; problem size scales exponentially with more scenarios. |
| Dynamic Programming (DP) | • Solves complex sequential dynamic problems globally without needing convexity. | • Suffers from the "Curse of Dimensionality" as the number of state variables increases. |
| Metaheuristics (Genetic Algorithm, Particle Swarm Optimization, Bayesian Optimization) | • Model-free; handles arbitrary complex, black-box, or derivative-free environments easily. | • No mathematical guarantee of global optimality. • Can be slow to converge and requires extensive parameter tuning. |
| Decomposition / Bilevel (Benders, Stackelberg) | • Drastically reduces problem scale. • Enables solving complex non-linear sizing/dispatch problems independently. | • Convergence depends heavily on algorithmic cuts/tuning. • Formulations can be difficult to implement. |

MILP has been used a lot of times in the literature to optimize different kinds of polygeneration systems. A similar study that researches the use of MILP to optimize a similar system of a hybrid biomass power-to-methanol system showed that MILP discovered optimal sizing and dispatch that results in LCOM of 805 EUR /t under the assumption of high biogas availability. This system adopts a configuration consisting of a low-pressure AWE, absorption-based CO2 capture, and steam production from methanol purge gas for internal heat supply [36]. In one energy-community case, MILP has been used to jointly optimize design and operation of distributed polygeneration with district heating and cooling, storage, and energy sharing, showing clear cost-emissions tradeoffs. The results showed two outcomes can come out depending on the objective function emphasis of either a 24.3% CO2 emission cut at a 1.9% cost increase or a 31.9% CO2 cut with a 2.22.2% CO2 emissions increase [37].

The key strength of MILP is the deterministic model that it offers on the modeled problem. If the system can be expressed with linear relationships, MILP guarantees an optimal solution to that formulation, which makes it highly attractive for sizing and dispatching problems with hard operational constraints [38]. Other than that, every constraint, feasibility, and infeasibility is explicit and can be traced to how the system is defined; thus, it has the benefit of easier interpretability and debugging. Due to the exactness and deterministic model, MILP becomes a great baseline for benchmarking novel methodologies such as RL [39].

The main gap of MILP is that optimality is guaranteed only for the approximated linear model, not necessarily the real physical system [40]. Therefore, in the case of complex polygeneration systems that contain nonlinear part-load efficiencies, thermal inertia, degradation, recycle interactions, and stochastic renewables, they often need to be simplified to their linear equivalent or linearized through processes such as piecewise linearization [41]. Another significant drawback is also the computation time that significantly increases with the number of non-convex definitions, decision variables, and long horizon, which is the exact opposite to a trained RL agent that can run in milliseconds after being trained, and results in a major limitation for real-time operation [42].

The reason why RL becomes interesting to explore is not that MILP is weak at optimization, rather that it is less suitable for fast repeated control when the environment is uncertain, partially observed, and dynamically changing [43], [44]. In biomass power-to-methanol systems, this would matter when biomass quality, renewable power input, hydrogen availability, market signals, and flexible downstream demand vary over time, forcing frequent re-optimization under nonlinear couplings. In this niche, RL literature positions itself as a complementary method rather than a direct replacement for MILP, and MILP remains a robust baseline to validate the results of this RL approach [45].

## State-of-the-art RL in Optimizing Energy and Industrial Systems

### Reinforcement Learning Fundamentals

The adoption of RL to optimize an energy system or industrial process has been of interest to a lot of researchers due to its compelling advantages, particularly due to its inherent flexibility and adaptability [46]. In comparison with MILP, RL is different because it acts as a sequential decision problem, which formulates control policies directly from interaction data rather than relying on a complete mathematical model [47]. The consensus among scholars concludes that RL is especially attractive where renewable variability, stochastic demand, and sector coupling make exact model-based control difficult [48]. Thus, a hybrid biomass power-to-methanol system is a great subject of analysis for the RL agent as it embodies all the characteristics mentioned.

By directly interacting with the system it is thrown into, the RL agent can derive optimal strategies through a series of Markov Decision Processes (MDPs). An MDP is described as a tuple {S, A, T, R}, where S denotes a set of states, A stands for a set of actions, T represents the transitional probability, and R denotes the reward function. This process involves a dynamic evolution of the states, actions, transitional probabilities, and reward at each time step, where the RL agent selects and executes an action at a current state and receives a reward from its action.

Figure 1 MDP process diagram.

The main objective of an agent is to maximize the discounted sum of rewards from time t to infinity, denoted by Rt , which is expressed as follows

Rt= Rt+1+ γRt+2+ γ2Rt+3+ …= k=0∞γkrt+k+1

Equation 10 Reward function of MDP process.

Where γ is a discount factor, specifying the future rewards. The agent's actions are guided by a policy π, which specifies what to do given the states of the environment. Each policy is associated with a state-value function Vπ(s), which predicts the outcome for state s when following policy π:

Vπs=ERtSt=s

Equation 11 State-value function.

Where E indicates the expected value. The optimal value of state s,  V*(s) , is defined to map the maximum value over all possible policies:

V*s=maxπ Et=0∞γtRst,ats0=s,at=πst

Equation 12 Optimal state-value function.

The other related variable is an action-value function Qπ(s,a), which gives the expected return when taking action a in state s and following policy π:

Qπs,a=ERtst=s,at=a

Equation 13 Action-value function.

The optimal Q-value of a state-action pair, Q*(s,a) is the maximum Q-value over all possible policies:

Q*s,a=max Eπt=0∞γtRst,ats0=s,a0=a,at>0=πst

Equation 14 Optimal action value function.

In short, an optimal policy π*(s) is a policy whose state-value function is equal to the optimal value of the state. Modelling an accurate environment’s dynamics through MDPs is emphasized by many researchers in the field to obtain a robust framework and good results [49]. Adopting this process enables RL agents to adapt to complex and dynamic environments effectively, which is ideal for optimizing the operation of a polygeneration plant [50].

To select an appropriate RL strategy for optimizing an energy system, it is paramount to understand the different structural arrangement that differentiates modern RL algorithms. RL algorithms are primarily categorized along two fundamental axes: (1) Model-based vs. Model-free RL, which specifies how the agent-environment dynamics, and (2) Value-Based, Policy-Based, and Hybrid Actor-Critic architectures, which determines how control policies are learned and updated. Depending on the type of system, data availability, and objective, one arrangement can be favored over the others.

### Environmental Dynamics of RL: Model-Based vs. Model-free

- Model-Based RL: The agent uses an explicit model of the system dynamics to plan future actions by simulating potential trajectories (e.g. Dyna-Q, Monte Carlo Tree Search, Model Predictive Path Integral control). In other words, the knowledge of the model directly influences the agent’s decision in choosing its next action. Although this approach introduces better sample efficiency, model-based RL often leads to longer online execution time and model-bias error which can be detrimental for highly complex and non-linear polygeneration system [].

- Model-Free RL: The agent treats the model of the system as a black box and purely relies on the navigating the accessible levers and system feedback to dictate its policy. As a consequence, this strategy requires significantly more data and longer training time for the agent to finally understand the underlying physics of the system. However, with a robust and accurately representing digital twin of the system, the agent can use the digital environment to generate as much data as it needs during the training period to finally reach a good policy while avoiding model-bias because all the physics are governed inside the digital twin black box [].

### Policy Optimization Algorithms: Value-Based, Policy-Based, and Actor-Critic

There are different types of RL algorithms to be chosen for complex energy system optimization and control. The types of RL algorithms are usually divided into three parts: value-based RL, policy-based RL, and actor-critic, which sits in between them. The difference between these different types lies in the learning methodology for the RL agent. Value-based RL learns a value function, usually a Q(s,a) estimate, and then chooses an action that returns the best estimated value. Some examples of value based RL are Q-learning and Deep Q Network (DQN) [51]. A study argues that value-function methods can run into convergence trouble, especially when the system is in continuous, multivariate settings. This algorithm excels in controlling and optimizing discrete choices but not continuous ones [52].

On the other hand, Policy-based RL learns the Policy directly, which means it parametrizes the action rule itself and pinpoints the optimal policy with methods like policy gradients and PPO [53]. There is much literature on RL that simply applies a value-based approach, however, since energy problem are often continuous, high-dimensional, and constrained, policy-based approaches are usually preferred [54].

Actor-critic methods are a mix of the two: the actor learns the policy, while the critic learns the estimated value to make training more stable. In the literature, earlier studies still adopted Q-learning, but newer research starts to implement actor-critic and other deep RL variants for complex control tasks [55]. This actor-critic method includes Proximal Policy Optimization (PPO), Soft Actor-Critic (SAC), Asynchronous Advantage Actor-Critic (A3C), Twin Delayed Deep Deterministic Policy Gradient (TD3), and Deep Deterministic Policy Gradient (DDPG). Numerous studies have shown the application of this algorithm method into different kinds of energy systems and concluded that it performs best when uncertainty, continuous control, and multi-timescale coordination matter [47], [56].

## RL vs MILP for Different Energy System Applications

In recent years, there have been multiple attempts in the literature to benchmark the results of RL and MILP for various energy system problems. Vetter et al. [26] analyzed the implementation of a deep reinforcement learning framework using the Deep Q-Network (DQN) algorithm to optimize the control of heat pump and heating element operation in a Grid-PV-Thermal Energy Storage (TES) system for an energy community. In this study, both frameworks are used to optimize only the dispatch strategy of the energy system in a fixed sizing environment. Furthermore, the RL agent’s action is only limited to controlling the SOC of the thermal energy storage, while observing its impact on the environment or state it can see. On the other hand, the MILP framework decides the flow of grid power and heating element at each time step. The result of this research shows that both frameworks successfully produce yearly cost savings compared to the reference scenario, with MILP having greater cost savings of a 10.06% reduction compared to 8.78% for RL. Aside from the cost result, the RL solution leads in terms of other metrics such as CO2 emission reduction, self-consumption ratio, and self-sufficiency ratio, without explicitly optimizing them. This is because RL decides to use more PV, which further reduces CO2 emissions.

Other than benchmarking the two frameworks for operational dispatch, some researchers also conducted a comparative analysis on RL and MILP for optimizing both the design and control decisions. Cauz et al. [57] used Battery-PV systems as their subject of analysis in order to test and optimize the performance of RL and MILP. The RL algorithm used here is the Direct Environment and Policy Search (DEPS). The design problem here lies in sizing the battery and PV power, whereas the control problem lies in dispatching battery and grid operations. The comparison is done in two stages: the first stage only involves one week of operation, and the second stage tests both frameworks’ performance in a one-year case study. For the first stage, the authors discovered that under a fixed sizing scenario, both frameworks manage to converge to the global optimum. However, while introducing the design decision problem, the RL at first managed to produce a better result in terms of cost; however, the authors later found that this is due to the modelling limitations that the RL excludes, which is the battery SOC balance at the end that should be the same as the initial SOC state. With this assumption being included in the calculation, the cost result of RL drops, and MILP prevails to become cheaper. This shows how important the modelling phase and identical assumptions are for a fair comparison. Interestingly, the RL approach manages to converge to a near-optimal solution with a design variable that is significantly different from the MILP: 62 kWh and 41 kWp for RL and 40 kWh and 103 kWp for MILP for battery and PV, respectively, with the RL layout having 145.65% of the cost of MILP after considering the additional SOC calculations.

In the second stage of this study, the RL approach failed to converge to the same cost value of MILP under the fixed sizing scenario, with it being 12% more expensive. Under the joint operational and sizing decision optimization scenario, RL ends up having 22% more cost than MILP, while the design decisions are significantly smaller than MILP (44 kWh & 57 kWp for RL and 95 kWh & 81 kWp for MILP for battery and PV, respectively). According to the authors of the study, this poorer performance of RL could be due to a lot of reasons; two of the notable reasons include hyperparameter tuning and the outdated RL algorithm chosen, which was developed in 1992.

Reviews now identify TD3, SAC, Multi-Agent Reinforcement Learning (MARL), and PPO as the main advanced RL families being adopted for energy management, especially in online or uncertainty-heavy settings [58]. As an on-policy policy-gradient framework, PPO utilizes a clipped surrogate objective function that constrains policy updates within a selected trust region, which ensures high training stability and steady convergence across continuous control domains [59]. In one study on optimizing the operation of multireservoir hydropower, Castro-Freibott et al. [60] implemented different RL algorithms (A2C, PPO, and SAC) in comparison to an MILP baseline. The experiment demonstrated that MILP achieves superior performance in the smaller system, but

PPO was the strongest RL method tested and scaled far better than MILP when the system gets bigger. This result also shows that PPO could maintain consistent performance across discrete and continuous action spaces while achieving it with less than one second of execution time. Another study compared four RL algorithms: PPO, A2C, SAC, TD3 for a battery power plant control problem and demonstrated TD3 superior results among the RL algorithms and outperformed MILP by 5% in cost savings [61]. In a study on optimizing renewables penetration in modern smart grids, a comparative analysis was conducted between rule-based, MILP, and DRL algorithms (DDPG, PPO, SAC). The result demonstrated SAC superiority in delivering the best results, reducing operational costs by 20%, achieving 92.8 % renewable application, while minimizing loss-of-load probability [62]. From diving deep into the literature, it is possible to argue that many RL strategies are already competitive with MILP, or even some better; however, this is highly dependent on the system complexity and uncertainty assumption.

# Methodology

## Hybrid Biomass Power-to-Methanol System Model

- Energy system definition

- Technical Parameters definition

- Economical Parameters definition

## Two-steps Outerloop & Innerloop Strategy

## Level 2 Optimization Architecture (MILP)

- Component level non-linear modelling logic

- Operation logic

  - What tool is used to model the MILP problem? Oemof.solph

  - What solver? CBC

- Objective function definition

## Level 3 Optimization Architecture (RL)

- Component level linear modelling logic

- Chosen RL Algorithm

- Operation logic (MDP)

  - How does BO and RL work together

- Objective function definition (Reward function)

## Data

## Comparison Stages & Scenario Definition

- This comparison will be done in two steps:

  - Outerloop comparison

    - The innerloop operational decision is fixed as an oemof problem,

    - for the outerloop (the process to select what layout to feed into the inner loop), different tools such as oemof investment block (IB), Bayesian Optimization (BO), and Design of Experiment (DOE) are compared

    - This outerloop comparison is to pinpoint which approach results in the best layout both under linear and non-linear economic of scale assumption.

    - Components to optimize: Electricity Supply & Power to X Optimization

  - Innerloop comparison:

    - two innerloop operational dispatch optimization strategies: RL agent living in a physics-based model of the plant vs oemof linear model of the plant are compared.

    - Stage 1: Fixed Outerloop: Linear Model with RL

      - How close can RL can optimize a linear model compared to level 2?

    - Stage 2: Fixed Outerloop: Non-Linear Model with RL

      - What happens to the optimized linear layout when it is injected into real physics-based model?

      - What happens when level 2 framework layout result is fed into level 3 model? Does it remain feasible? And is it still a good solution?

      - What is the difference between the result of simplified linear model vs complex nonlinear model?

    - Stage 3: Both outerloop & Innerloop BO RL Nonlinear vs BO Oemof Linear pathway

      - How different is the optimal results of a simplified linear model and accurate complex physics-based model?

- There are four case studies for each step to be analyzed (For innerloop 3 comparison):

  - Weather Variability:

    - Stable solar and wind input

    - Extreme weather volatility

  - Market Conditions:

    - Cheap batteries

    - Cheap hydrogen system (or maybe -50%, -20%, +20%, +50%)

  - System Design & Flexibility:

    - Heavy fixed methanol reactor load

    - Heavy fixed electrolyzer load

- For each of the steps, stages, and case studies, there are some predetermined KPIs to be examined:

  - Techno-economic level:

    - LCOM

    - CAPEX & OPEX

    - Carbon intensity

    - Grid dependency ratio

    - Self-consumption Ratio

    - Autonomy days

  - Framework level:

    - Simulation time

    - BO Distance

  - Scenario:

  - Year:

  - CAPEX and OPEX per component:

  - Conversion Factor per component:

  - Fixed components:

    - MeOH reactor

    - Distillery

    - Storage MeOH

    - Anaerobic Digester

    - ATR

    - O2 Storage: Full load of ATR sizing x3 days

    - Steam generator

  - AUX:

    - RWGS

    - Fuel Boiler

    - AD Gas Separator (Size depends on )

    - Compressor

    - 3 Heat Exchangers

  - Variable components: (IB vs BO vs DOE)

    - CH4 Generator or GT

    - Wind turbine

    - PV

    - BESS

    - Electrolyzer

    - Hydrogen storage

    - Biogas storage

# Results and Discussions

# Conclusions

# References

[1]	S. Paltsev, J. Morris, H. Kheshgi, and H. Herzog, “Hard-to-Abate Sectors: The role of industrial carbon capture and storage (CCS) in emission mitigation,” Appl. Energy, vol. 300, p. 117322, Oct. 2021, doi: 10.1016/j.apenergy.2021.117322.

[2]	J. Sousa, I. Azevedo, C. Camus, L. Mendes, C. Viveiros, and F. Barata, “Decarbonizing Hard-to-Abate Sectors with Renewable Hydrogen: A Real Case Application to the Ceramics Industry,” Energies (Basel)., vol. 17, no. 15, p. 3661, Jul. 2024, doi: 10.3390/en17153661.

[3]	A. Risco-Bravo, C. Varela, J. Bartels, and E. Zondervan, “From green hydrogen to electricity: A review on recent advances, challenges, and opportunities on power-to-hydrogen-to-power systems,” Renewable and Sustainable Energy Reviews, vol. 189, p. 113930, Jan. 2024, doi: 10.1016/j.rser.2023.113930.

[4]	Y. Yao et al., “A comprehensive review of optimizing hybrid renewable energy system using optimization techniques,” Engineering Science and Technology, an International Journal, vol. 78, p. 102384, Jun. 2026, doi: 10.1016/j.jestch.2026.102384.

[5]	P. Wohlgenannt, S. Hegenbart, E. Eder, M. Kolhe, and P. Kepplinger, “Energy Demand Response in a Food-Processing Plant: A Deep Reinforcement Learning Approach,” Energies (Basel)., vol. 17, no. 24, p. 6430, Dec. 2024, doi: 10.3390/en17246430.

[6]	R. Yokoyama, Y. Shinano, S. Taniguchi, M. Ohkura, and T. Wakui, “Optimization of energy supply systems by MILP branch and bound method in consideration of hierarchical relationship between design and operation,” Energy Convers. Manag., vol. 92, pp. 92–104, Mar. 2015, doi: 10.1016/j.enconman.2014.12.020.

[7]	S. Stavrev and D. Ginchev, “Reinforcement Learning Techniques in Optimizing Energy Systems,” Electronics (Basel)., vol. 13, no. 8, p. 1459, Apr. 2024, doi: 10.3390/electronics13081459.

[8]	H. Ghiasirad et al., “LCA and exergy-economic evaluation of a biogas-to-fuels system using CO2 hydrogenation and exhaust gas recycling,” Energy Conversion and Management: X, vol. 26, p. 100897, Apr. 2025, doi: 10.1016/j.ecmx.2025.100897.

[9]	A. Grimalt‐Alemany, I. V. Skiadas, and H. N. Gavala, “Syngas biomethanation: state‐of‐the‐art review and perspectives,” Biofuels, Bioproducts and Biorefining, vol. 12, no. 1, pp. 139–158, Jan. 2018, doi: 10.1002/bbb.1826.

[10]	M. Scomazzon, E. Barbera, and F. Bezzo, “Alternative sustainable routes to methanol production: Techno-economic and environmental assessment,” J. Environ. Chem. Eng., vol. 12, no. 3, p. 112674, Jun. 2024, doi: 10.1016/j.jece.2024.112674.

[11]	E. Moioli and T. Schildhauer, “Techno-economic-environmental assessment of the integration of power-to-X and biogas utilization towards the production of electricity, hydrogen, methane and methanol,” Sustain. Energy Fuels, vol. 7, no. 11, pp. 2690–2706, 2023, doi: 10.1039/D3SE00052D.

[12]	H. Zhang, L. Wang, J. Van herle, F. Maréchal, and U. Desideri, “Techno-economic evaluation of biomass-to-fuels with solid-oxide electrolyzer,” Appl. Energy, vol. 270, p. 115113, Jul. 2020, doi: 10.1016/j.apenergy.2020.115113.

[13]	A. Walther, K. Löning, and E. Reichelt, “Techno-economic assessment of the impact of electrolysis technology on power-to-methanol processes,” Int. J. Hydrogen Energy, vol. 206, p. 153394, Feb. 2026, doi: 10.1016/j.ijhydene.2026.153394.

[14]	J. Brauns and T. Turek, “Alkaline Water Electrolysis Powered by Renewable Energy: A Review,” Processes, vol. 8, no. 2, p. 248, Feb. 2020, doi: 10.3390/pr8020248.

[15]	S. Brynolf, M. Taljegard, M. Grahn, and J. Hansson, “Electrofuels for the transport sector: A review of production costs,” Renewable and Sustainable Energy Reviews, vol. 81, pp. 1887–1905, Jan. 2018, doi: 10.1016/j.rser.2017.05.288.

[16]	S. Shiva Kumar and V. Himabindu, “Hydrogen production by PEM water electrolysis – A review,” Mater. Sci. Energy Technol., vol. 2, no. 3, pp. 442–454, Dec. 2019, doi: 10.1016/j.mset.2019.03.002.

[17]	H. Ghiasirad et al., “LCA and exergy-economic evaluation of a biogas-to-fuels system using CO2 hydrogenation and exhaust gas recycling,” Energy Conversion and Management: X, vol. 26, p. 100897, Apr. 2025, doi: 10.1016/j.ecmx.2025.100897.

[18]	M. Scomazzon, E. Barbera, and F. Bezzo, “Alternative sustainable routes to methanol production: Techno-economic and environmental assessment,” J. Environ. Chem. Eng., vol. 12, no. 3, p. 112674, Jun. 2024, doi: 10.1016/j.jece.2024.112674.

[19]	E. Moioli and T. Schildhauer, “Techno-economic-environmental assessment of the integration of power-to-X and biogas utilization towards the production of electricity, hydrogen, methane and methanol,” Sustain. Energy Fuels, vol. 7, no. 11, pp. 2690–2706, 2023, doi: 10.1039/D3SE00052D.

[20]	M. Dossow, V. Dieterich, A. Hanel, H. Spliethoff, and S. Fendt, “Improving carbon efficiency for an advanced Biomass-to-Liquid process using hydrogen and oxygen from electrolysis,” Renewable and Sustainable Energy Reviews, vol. 152, p. 111670, Dec. 2021, doi: 10.1016/j.rser.2021.111670.

[21]	A. Poluzzi et al., “Flexible Power and Biomass-To-Methanol Plants With Different Gasification Technologies,” Front. Energy Res., vol. 9, Jan. 2022, doi: 10.3389/fenrg.2021.795673.

[22]	E. Anetjärvi, E. Vakkilainen, and K. Melin, “Benefits of Hybrid Production of E-Methanol in Connection with Biomass Gasification,” SSRN Electronic Journal, 2022, doi: 10.2139/ssrn.4184476.

[23]	H. Zhang, L. Wang, M. Pérez-Fortes, J. Van herle, F. Maréchal, and U. Desideri, “Techno-economic optimization of biomass-to-methanol with solid-oxide electrolyzer,” Appl. Energy, vol. 258, p. 114071, Jan. 2020, doi: 10.1016/j.apenergy.2019.114071.

[24]	A. Poluzzi et al., “Flexible Power &amp; Biomass-to-Methanol plants: Design optimization and economic viability of the electrolysis integration,” Fuel, vol. 310, p. 122113, Feb. 2022, doi: 10.1016/j.fuel.2021.122113.

[25]	A. Poluzzi et al., “Flexible Power &amp; Biomass-to-Methanol plants: Design optimization and economic viability of the electrolysis integration,” Fuel, vol. 310, p. 122113, Feb. 2022, doi: 10.1016/j.fuel.2021.122113.

[26]	V. Vetter, P. Wohlgenannt, P. Kepplinger, and E. Eder, “Deep Reinforcement Learning Approaches the MILP Optimum of a Multi-Energy Optimization in Energy Communities,” Energies (Basel)., vol. 18, no. 17, p. 4489, Aug. 2025, doi: 10.3390/en18174489.

[27]	P. Wohlgenannt, S. Hegenbart, E. Eder, M. Kolhe, and P. Kepplinger, “Energy Demand Response in a Food-Processing Plant: A Deep Reinforcement Learning Approach,” Energies (Basel)., vol. 17, no. 24, p. 6430, Dec. 2024, doi: 10.3390/en17246430.

[28]	M. Thirunavukkarasu, Y. Sawle, and H. Lala, “A comprehensive review on optimization of hybrid renewable energy systems using various optimization techniques,” Renewable and Sustainable Energy Reviews, vol. 176, p. 113192, Apr. 2023, doi: 10.1016/j.rser.2023.113192.

[29]	H. Shengren, P. P. Vergara, E. M. Salazar Duque, and P. Palensky, “Optimal energy system scheduling using a constraint-aware reinforcement learning algorithm,” International Journal of Electrical Power & Energy Systems, vol. 152, p. 109230, Oct. 2023, doi: 10.1016/j.ijepes.2023.109230.

[30]	A. Jaber, R. Younes, P. Lafon, and J. Khoder, “A Review on Multi-Objective Mixed-Integer Non-Linear Optimization Programming Methods,” Eng, vol. 5, no. 3, pp. 1961–1979, Aug. 2024, doi: 10.3390/eng5030104.

[31]	B. Pirouz and F. Guerriero, “Multi-objective stochastic optimization problem: a systematic literature review,” Appl. Energy, vol. 405, p. 127237, Feb. 2026, doi: 10.1016/j.apenergy.2025.127237.

[32]	S. Basnet, K. Deschinkel, L. Le Moyne, and M. Cécile Péra, “A review on recent standalone and grid integrated hybrid renewable energy systems: System optimization and energy management strategies,” Renewable Energy Focus, vol. 46, pp. 103–125, Sep. 2023, doi: 10.1016/j.ref.2023.06.001.

[33]	T. Kleinert, M. Labbé, I. Ljubić, and M. Schmidt, “A Survey on Mixed-Integer Programming Techniques in Bilevel Optimization,” EURO Journal on Computational Optimization, vol. 9, p. 100007, 2021, doi: 10.1016/j.ejco.2021.100007.

[34]	I. E. Grossmann, R. M. Apap, B. A. Calfa, P. García-Herreros, and Q. Zhang, “Recent advances in mathematical programming techniques for the optimization of process systems under uncertainty,” Comput. Chem. Eng., vol. 91, pp. 3–14, Aug. 2016, doi: 10.1016/j.compchemeng.2016.03.002.

[35]	C. Li and I. E. Grossmann, “A Review of Stochastic Programming Methods for Optimization of Process Systems Under Uncertainty,” Frontiers in Chemical Engineering, vol. 2, Jan. 2021, doi: 10.3389/fceng.2020.622241.

[36]	P. Kenkel, C. Schnuelle, T. Wassermann, and E. Zondervan, “Integrating multi-objective superstructure optimization and multi-criteria assessment: a novel methodology for sustainable process design,” Physical Sciences Reviews, vol. 8, no. 9, pp. 2361–2394, Sep. 2023, doi: 10.1515/psr-2020-0058.

[37]	R. J. De Souza, M. Reini, L. M. Serra, M. A. Lozano, E. Nadalon, and M. Casisi, “Multi-Objective Optimization of an Energy Community Powered by a Distributed Polygeneration System,” Energies (Basel)., vol. 17, no. 13, p. 3085, Jun. 2024, doi: 10.3390/en17133085.

[38]	P. Wohlgenannt, S. Hegenbart, E. Eder, M. Kolhe, and P. Kepplinger, “Energy Demand Response in a Food-Processing Plant: A Deep Reinforcement Learning Approach,” Energies (Basel)., vol. 17, no. 24, p. 6430, Dec. 2024, doi: 10.3390/en17246430.

[39]	H. Shengren, E. M. Salazar, P. P. Vergara, and P. Palensky, “Performance Comparison of Deep RL Algorithms for Energy Systems Optimal Scheduling,” in 2022 IEEE PES Innovative Smart Grid Technologies Conference Europe (ISGT-Europe), IEEE, Oct. 2022, pp. 1–6. doi: 10.1109/ISGT-Europe54678.2022.9960642.

[40]	C. Huang, H. Zhang, L. Wang, X. Luo, and Y. Song, “Mixed Deep Reinforcement Learning Considering Discrete-continuous Hybrid Action Space for Smart Home Energy Management,” Journal of Modern Power Systems and Clean Energy, vol. 10, no. 3, pp. 743–754, 2022, doi: 10.35833/MPCE.2021.000394.

[41]	W. Khan et al., “Deep reinforcement learning-based energy management for design and control of off-grid renewable microgrids with dual-battery storage,” Energy Exploration & Exploitation, vol. 44, no. 2, pp. 821–869, Mar. 2026, doi: 10.1177/01445987251387368.

[42]	P. Wohlgenannt, S. Hegenbart, E. Eder, M. Kolhe, and P. Kepplinger, “Energy Demand Response in a Food-Processing Plant: A Deep Reinforcement Learning Approach,” Energies (Basel)., vol. 17, no. 24, p. 6430, Dec. 2024, doi: 10.3390/en17246430.

[43]	S. Stavrev and D. Ginchev, “Reinforcement Learning Techniques in Optimizing Energy Systems,” Electronics (Basel)., vol. 13, no. 8, p. 1459, Apr. 2024, doi: 10.3390/electronics13081459.

[44]	V. Vetter, P. Wohlgenannt, P. Kepplinger, and E. Eder, “Deep Reinforcement Learning Approaches the MILP Optimum of a Multi-Energy Optimization in Energy Communities,” Energies (Basel)., vol. 18, no. 17, p. 4489, Aug. 2025, doi: 10.3390/en18174489.

[45]	H. Shengren, P. P. Vergara, E. M. Salazar Duque, and P. Palensky, “Optimal energy system scheduling using a constraint-aware reinforcement learning algorithm,” International Journal of Electrical Power & Energy Systems, vol. 152, p. 109230, Oct. 2023, doi: 10.1016/j.ijepes.2023.109230.

[46]	S. Stavrev and D. Ginchev, “Reinforcement Learning Techniques in Optimizing Energy Systems,” Apr. 01, 2024, Multidisciplinary Digital Publishing Institute (MDPI). doi: 10.3390/electronics13081459.

[47]	A. T. D. Perera and P. Kamalaruban, “Applications of reinforcement learning in energy systems,” Renewable and Sustainable Energy Reviews, vol. 137, p. 110618, Mar. 2021, doi: 10.1016/j.rser.2020.110618.

[48]	S. Stavrev and D. Ginchev, “Reinforcement Learning Techniques in Optimizing Energy Systems,” Electronics (Basel)., vol. 13, no. 8, p. 1459, Apr. 2024, doi: 10.3390/electronics13081459.

[49]	J. Langford, “Efficient Exploration in Reinforcement Learning,” in Encyclopedia of Machine Learning, Boston, MA: Springer US, 2011, pp. 309–311. doi: 10.1007/978-0-387-30164-8_244.

[50]	Y. Liu, J. Yang, L. Chen, T. Guo, and Y. Jiang, “Overview of Reinforcement Learning Based on Value and Policy,” in 2020 Chinese Control And Decision Conference (CCDC), IEEE, Aug. 2020, pp. 598–603. doi: 10.1109/CCDC49329.2020.9164615.

[51]	R. Lincoln, S. Galloway, B. Stephen, and G. Burt, “Comparing Policy Gradient and Value Function Based Reinforcement Learning Methods in Simulated Electrical Power Trade,” IEEE Transactions on Power Systems, vol. 27, no. 1, pp. 373–380, Feb. 2012, doi: 10.1109/TPWRS.2011.2166091.

[52]	Y. Gao, “Policy Gradient Methods in Deep Reinforcement Learning,” Applied and Computational Engineering, vol. 158, no. 1, pp. 27–34, May 2025, doi: 10.54254/2755-2721/2025.TJ23321.

[53]	D. Cao et al., “Reinforcement Learning and Its Applications in Modern Power and Energy Systems: A Review,” Journal of Modern Power Systems and Clean Energy, vol. 8, no. 6, pp. 1029–1042, 2020, doi: 10.35833/MPCE.2020.000552.

[54]	H. Shengren, E. M. Salazar, P. P. Vergara, and P. Palensky, “Performance Comparison of Deep RL Algorithms for Energy Systems Optimal Scheduling,” in 2022 IEEE PES Innovative Smart Grid Technologies Conference Europe (ISGT-Europe), IEEE, Oct. 2022, pp. 1–6. doi: 10.1109/ISGT-Europe54678.2022.9960642.

[55]	M. Cauz, A. Bolland, B. Miftari, L. Perret, C. Ballif, and N. Wyrsch, “Reinforcement Learning for Joint Design and Control of Battery-PV Systems,” Jul. 2023, [Online]. Available: http://arxiv.org/abs/2307.04244