# Optimization Frameworks for Hybrid Biomass Power-to-Methanol System: MILP and Reinforcement Learning Approaches

# Muhammad Rafi Afif Indrajaya

Erasmus Mundus Decentralized Smart Energy System (DENSYS)

# Politecnico di Torino

Submitted on July, 18th 2026

# Abstract

# Acknowledgment

I want to ddedicate this thesis for abladlkafjldkjfal flkad ladshflasdhf lasdkhfasd hflksadhflak hflkadhflkas dhflkasdh flkadshflkasdhflhadlsf hsadlf hadslfh lsadkh flskadhf lkasdhf ldkshf lksdahf lkdashf dss fsd fsdf sdf sdf sdfs df sdfaf asf asf asdf asdf as

Sd fasd fsadf

Fasdf asdf asf asd

Ds afda fsad f

D fas fasdf asdf

Dsf asdf adsf asdf

Df asdf asf

A dfasdf as

Sfsdfsdfadfasdfasdfadsfasdfasf

Dsfadsdfasd

Sdafdasf

Sdafasdf

Fsgsfgdsgsdfg

Dsfgfsgfsd

Gsdfgsdfgfsdgf

Sdfgsfdgsdfg

Sdfgsdfgsdf

Gsdfgsdfgsdf

gsdfgsdfgsdfgdsgdsfg

Table of Contents

1	Introduction	1

1.1	Background	1

1.2	Introduction to Company	2

1.3	Objective	3

1.4	Thesis Configuration	4

2	Literature Review	5

2.1	Power-to-X Integration for Methanol Synthesis System	5

2.2	MILP for Complex Polygeneration System	7

2.3	State-of-the-art RL in Optimizing Energy System and Industrial Processes	8

2.4	RL vs MILP for Different Applications	10

3	Methodology	13

3.1	Hybrid Biomass Power-to-Methanol System Model	13

3.2	Two-steps Outerloop & Innerloop Strategy	13

3.3	Level 2 Optimization Architecture (MILP)	13

3.4	Level 3 Optimization Architecture (RL)	13

3.5	Data	13

3.6	Comparison Stages & Scenario Definition	13

4	Results and Discussions	17

4.1	17

5	Conclusions	18

6	References	19

# List of Abbreviations

# Introduction

## Background

The large-scale transformation of industrial energy systems is widely regarded as a critical enabler for achieving global decarbonization targets, particularly in so-called hard-to-abate sectors [1]. Among the available pathways, Power-to-X (PtX) technologies have emerged as a promising solution, enabling the conversion of renewable electricity into chemical energy carriers and synthetic fuels suitable for industrial applications [2]. However, PtX systems are inherently complex and capital-intensive, especially when their operation is coupled with the variability and intermittency of renewable energy source [3]. This complexity introduces significant challenges in both system design and operational planning, thereby necessitating robust optimization strategies that can effectively minimize capital expenditures (CAPEX) and operational expenditures (OPEX).

To address this need, Mixed-Integer Linear Programming (MILP) has become the prevailing industrial standard due to its robustness, transparency, and well-established solution techniques [4]. At the same time, the rapid advancement of artificial intelligence (AI) has motivated the exploration of alternative methodologies, particularly reinforcement learning (RL) [5]. In this context, RL can be understood as an agent-based framework that learns optimal operational strategies through iterative interaction with the system environment, effectively acting as an autonomous plant operator.

Despite its widespread adoption, MILP relies on linear approximations of system behavior, which can limit model fidelity when representing inherently nonlinear processes [6]. In practice, such nonlinearities are often simplified or neglected. This limitation makes RL particularly attractive, as it allows for flexible environment design in which high-fidelity, nonlinear system dynamics can be directly incorporated. Through appropriately defined reward structures, the RL agent can learn operational strategies that account for these complexities without requiring explicit linearization [7].

Building on this motivation, this thesis adopts a two-step methodological framework to systematically compare different levels of modeling fidelity and optimization approaches. In the first step, a comparison is conducted between a fully integrated MILP formulation and a decoupled framework in which system design (sizing) and operation (dispatch) are optimized separately. This step aims to assess the impact of incorporating nonlinear CAPEX assumptions in the outer-loop (design optimization). In the second step, the resulting optimal system sizing is used as an input for operational optimization, where both MILP and RL-based approaches are applied in the inner-loop (dispatch optimization). This structure enables a direct comparison of the two frameworks in terms of their ability to determine cost-optimal operational strategies, particularly when contrasting linearized system representations with higher-fidelity nonlinear models.

In summary, this thesis evaluates the techno-economic performance of a reinforcement learning–based approach against a conventional MILP-based framework for a hybrid biomass power-to-methanol plant located in South Africa. The analysis is performed across multiple scenarios and case studies to capture a range of operating conditions and system configurations. Through this comparative assessment, the study seeks to clarify the role and current capabilities of RL in optimizing complex Power-to-X systems, while benchmarking its performance against an established industrial standard.

## Introduction to Company

This master’s thesis is conducted in cooperation with FEV Europe GmbH, specifically within its energy+resources department. In this industrial context, the research aligns with ongoing efforts to expand capabilities beyond traditional automotive engineering toward energy-related applications.

FEV Europe GmbH is widely recognized as an established expert within the automotive industry, particularly for its benchmarking and consulting services. Building on this foundation, the company is increasingly directing its efforts toward supporting innovation within the energy sector, reflecting broader industry trends and internal strategic priorities.

Within this framework, the present thesis contributes to the identification and evaluation of an efficient optimization tool for innovative energy solutions, with a focus on complex polygeneration plants. The outcomes are intended to support future FEV Europe GmbH projects by enabling improved system design and operational decision-making.

## Objective

This thesis aims to systematically evaluate and compare two optimization frameworks across multiple operational scenarios. The analysis focuses on assessing their ability to represent and optimize complex hybrid energy systems, with particular attention to differences in system configuration and performance outcomes.

A central objective is to investigate whether a non-linear physical model integrated with a reinforcement learning (RL) agent converges to substantially different component sizing and system layouts compared to a mixed-integer linear programming (MILP) approach. These differences are examined in terms of key techno-economic indicators, including capital expenditure (CAPEX), operational expenditure (OPEX), clipped energy, simulation time, amount of methanol produced and levelized cost of methanol (LCOM).

Based on the simulation results, the thesis further aims to identify and explain the underlying reasons for any observed discrepancies—or the lack thereof—between the two frameworks. This comparative analysis supports a final assessment of whether one framework demonstrates superior performance or whether its suitability is dependent on specific system configurations and operating conditions.

The expected contribution of this work is to enable the selection of an appropriate optimization framework for hybrid methanol synthesis plants, while also providing a structured approach for framework selection in other industrial energy systems. Additionally, the findings aim to improve optimization accuracy and/or reduce computational effort, and to enhance understanding of where and how RL-based approaches can be effectively applied within polygeneration system design.

## Thesis Configuration

This thesis is structured to provide a systematic and transparent comparison of the selected optimization frameworks. Chapter 2 reviews the state-of-the-art literature on the application of reinforcement learning (RL) in optimizing complex industrial processes and energy systems, including existing comparative studies between optimization approaches.

Chapter 3 details the methodological framework adopted for the comparison. It outlines a multi-stage evaluation procedure, describing each step taken to ensure a consistent and rigorous assessment of the two optimization approaches.

Chapter 4 presents and discusses the results of this multi-stage comparison. It analyzes the performance of each framework across different scenarios and case studies, and introduces a generalization scoring approach to assess their robustness and applicability across multiple evaluation dimensions.

Finally, Chapter 5 summarizes the main findings of the thesis and reflects on key limitations, assumptions, and methodological considerations that underpin the analysis, providing context for the interpretation and transferability of the results.

# Literature Review

## Hybrid Biomass Power-to-Methanol System

The hybridization strategy considered in this thesis integrates two upstream pathways that jointly supply the feedstock for methanol synthesis: (i) a biomass-derived pathway and (ii) a hydrogen pathway based on water electrolysis. This configuration enables coordinated carbon and hydrogen management prior to the synthesis loop.

In the biomass pathway, available feedstocks are converted into either syngas or biogas, which is subsequently upgraded to syngas for methanol production. In the literature, gasification is most commonly employed to directly convert solid biomass into syngas. An alternative route relies on anaerobic digestion (AD), where biomass is first converted into biogas and then reformed to syngas prior to methanol synthesis. The differences in process steps primarily drive the trade-offs between these technologies. The AD-based route is attractive because it can be integrated with existing waste-treatment infrastructure; however, it requires additional downstream conversion steps, making the pathway less direct and more dependent on reforming, CO₂ handling, and potentially electrolysis integration. In contrast, gasifier-based routes are more established for methanol synthesis, as they provide syngas directly and are well suited for coupling with high-temperature processes. Their disadvantages include challenges related to high-temperature solid handling, the requirement for an oxygen supply (e.g., via an air separation unit, ASU), and stricter gas-cleaning requirements [8].

Table 1 Structural and Process Comparison between AD and Gasification routes

| Metric | Anaerobic Digestion (AD) Route | Biomass Gasification Route | References |
| --- | --- | --- | --- |
| Primary Conversion Mechanism | Biochemical (mesophilic/thermophilic digestion of wet organic feedstocks) | Thermochemical high-temperature reactions (>800∘C-1200∘C) | [9] |
| Intermediate Stream | Biogas (∼55--65% CH4, 35--45% CO2) requiring reforming to form syngas. | Raw syngas (CO,H2,CO2,CH4) containing tars and particulates. | [9], [10] |
| Feedstock Flexibility | High-moisture, biodegradable substrates (e.g., sewage sludge, organic municipal solid waste, agricultural slurry). | Low-moisture solid biomass (e.g., forestry residues, lignocellulosic crop waste, refuse-derived fuel/RDF). | [9] |
| Core Process Bottleneck | Biogas reforming (Dry Reforming DRM / Steam Reforming SMR) and stoichiometric H2/CO ratio adjustment. | Severe syngas conditioning requirements (tar removal, particulate scrubbing, sulfur/halogen cleanup, and Water-Gas Shift). | [9], [11] |
| Techno-Economic Profile | Lower CAPEX per unit capacity; economically viable at smaller, regional plant sizes. | High upfront CAPEX; strongly dependent on economies of scale and centralized feedstock supply chains. | [10] |
| Lifecycle GHG Abatement | Moderate-to-high carbon footprint reduction depending on feedstock cultivation and digestate handling. | Exceptional carbon abatement potential (up to -0.88 t CO2e/t CH3OH) when combined with biogenic carbon capture. | [10] |

The hydrogen pathway begins with the electrochemical splitting of water into hydrogen and oxygen using an electrolyzer. Several studies identify Solid Oxide Electrolyzer Cells (SOECs) as a promising option for power-to-methanol systems. The high operating temperature of SOECs allows for heat integration and potential oxygen utilization, which can increase overall plant efficiency compared to low-temperature systems[12]. However, key challenges include long-term stack durability, technological maturity, and limited operational flexibility, which is particularly relevant for systems coupled to variable renewable electricity. Techno-economic comparisons suggest that alkaline water electrolysis (AWE) represents a viable short- to mid-term option, whereas SOECs are more likely to be competitive in the mid- to long-term. Low-temperature electrolyzers such as AWE and proton exchange membrane (PEM) systems offer higher technological maturity and superior load-following capability, which is advantageous for renewable integration, but they do not fully exploit heat integration or co-electrolysis opportunities available to SOECs [13]. A detailed quantitative synthesis of these performance metrics and operating limits is summarized in Table 2.

Table 2 Parameter comparison between AWE, PEM, and SOEC [14], [15], [16].

| Parameter | Alkaline Water Electrolysis (AWE) | Proton Exchange Membrane (PEM) | Solid Oxide Electrolysis (SOEC) |
| --- | --- | --- | --- |
| Operating Temperature (​∘C) | 60-90∘C | 50-80∘C | 650-1000∘C |
| Operating Pressure (bar) | <30 bar | 30-80 bar | <30 bar |
| Energy Consumption (kWh/Nm3 H2) | 4.5-5.5 | 4.2-5.5 | 3.2-3.8 |
| Minimum Operational Load (%Pnom) | 10%-20% | 0%-10% | 20%-30% |
| Dynamic Response | Moderate (0.2--20%/s) | Very High (10--100%/s) | Low (<1%/s) |
| H2 Purity (%) | >99.5% | >99.999% | >99.9% |
| Commercial Status / TRL | Commercial / State-of-the-art (TRL 9) | Near-commercial (TRL 8-9) | R&D (TRL 6-7) |

In the integrated configuration, the electrolyzer produces both hydrogen and oxygen. Hydrogen is compressed and supplied to the methanol synthesis reactor, while the co-produced oxygen can be utilized in an autothermal reformer (ATR) to convert methane (e.g., from biogas) into syngas. The biomass-derived and electrolysis-derived streams are thus combined upstream of, or within, the methanol synthesis loop. [NEED REFERENCE: process integration scheme describing oxygen use in ATR and hydrogen blending in methanol synthesis]

The hybrid approach to methanol production is attractive due to its potential for higher carbon utilization and improved use of biomass resources. One study reports a carbon utilization of 89.8% and an overall efficiency of 85.8%, with up to 101% higher methanol production per unit of biomass compared to a conventional biomass-to-methanol baseline (e.g., https://dx.doi.org/10.2139/ssrn.4184476). Another analysis indicates that a state-of-the-art biomass-to-methanol process achieves approximately 53.3% efficiency with a 4.8-year payback period, while integrating electrolysis for full carbon conversion increases efficiency to 64.5–66.0%, albeit with less favorable economics (e.g., https://doi.org/10.1016/j.apenergy.2019.114071). Furthermore, under favorable electricity price conditions, flexible operation can allow the plant to capture value from temporal price variations instead of operating continuously at a fixed load (e.g., https://doi.org/10.1016/j.fuel.2021.122113).

Despite these advantages, several bottlenecks challenge the feasibility of hybrid methanol systems. These include electrolyzer utilization rates, electricity price volatility, capital cost and lifetime of electrolyzer stacks, as well as the integration of oxygen and heat streams. For example, it has been reported that high electrolyzer capacity factors are required for competitive e-methanol production, and that full-carbon-conversion configurations become attractive only when improvements in electricity costs, stack lifetime, and capital expenditure are realized (e.g., https://doi.org/10.1016/j.fuel.2021.122113). Oxygen management represents an additional practical constraint: while direct integration of oxygen streams may reduce the need for large-scale oxygen storage and improve capital efficiency and safety, this is contingent on matching oxygen supply and demand within the plant (e.g., https://doi.org/10.3389/fenrg.2021.795673). [NEED REFERENCE: quantitative analysis of oxygen balancing and storage requirements in hybrid systems]

To address these challenges, process and system-level optimization emerges as a critical tool for improving both technical and economic performance. For instance, optimization of the methanol synthesis reactor in a power-and-biomass-to-methanol (PBtM) configuration has been shown to reduce e-methanol production costs by approximately 20% (e.g., https://doi.org/10.1016/j.fuel.2021.122113). While such studies typically focus on individual subsystems, it is reasonable to expect that plant-wide optimization could yield further cost reductions and efficiency gains. [NEED REFERENCE: system-level optimization studies demonstrating integrated plant improvements] In this context, optimization acts as the bridge between technically feasible system designs and configurations that remain viable under realistic operating conditions, including fluctuating electricity prices, electrolyzer degradation, and large-scale capital constraints.

## MILP for Complex Polygeneration System

- Introduction of MILP as the state-of-the-art methodology for optimizing various energy system by defining linear models, constraints and objective functions

- What other methodologies that are used that are adjacent to MILP

- Strengths and gaps of MILP compared to the other adjacent methodologies

- MILP use cases in optimizing hybrid methanol system or other polygeneration or specifically hybrid biomass power-to-methanol system

- The gap that MILP has that incentivizes the use of RL for optimizing a complex polygeneration system

MILP, or Mixed-Integer Linear Programming, has been used as a tool to optimize different kinds of energy or industrial plants by linearizing the model of these processes and setting up a linear objective function to either maximize or minimize a specific KPI based on the optimization objective []. In an energy system that includes the flow of electricity, heat, and mass, the constraints considered will then dictate how the balance of the system should behave, the minimum and maximum operational dispatch, and the evolution of the storage state of charge, if storage exists.

- Equations considered in the MILP formulation

On the other hand, the objective function mainly includes the capital expenditure (CAPEX) and operational expenditure (OPEX) of the models defined in the system.

- How CAPEX and OPEX is defined in MILP as linear equations

Together, the MILP will generate a global equation that accounts for the constraints and the objective function, and, using the MILP solver, it will perform relaxation and branch-and-bound to find the valleys or hills within the linear boundaries. In practice, MILP has been used as the reference method for operational optimization, demand response, scheduling in energy communities, industrial plants, and integrated multi-energy systems because it produces globally optimal solutions to the defined linear problem when sufficient computation time is available [17].

The main methodological alternatives adjacent to MILP are MINLP, stochastic programming, dynamic programming, metaheuristics, and decomposition-based or bilevel formulations [18]. MINLP is utilized when the non-linear physics of the assigned system is prevalent, but as the system gets bigger and more variables are involved, the problem can grow exponentially to the point where it is extremely difficult to solve. Stochastic programming addresses uncertainty by executing the system in many scenarios, but its computational burden rises sharply with scenario count and system size. Metaheuristics such as Particle Swarm Optimization (PSO) and Genetic Algorithm (GA) are widely used for non-linear multiobjective hybrid energy problem due to its flexibility and robustness, however, they do not guarantee the global optamility in comparison to the MILP counterpart. There is also a hybrid approach that is usually called a bilevel formulation, where it separates the optimization process of sizing selection using a metaheuristic approach and operational dispatch optimization using approach like MILP or RL. This will be discussed even further in depth in another sub-section.

- Table of main advantage and disadvantage of different optimization approaches

MILP has been used a lot of times in the literature to optimize different kinds of polygeneration system. A similar study that researches the use of MILP to optimize a similar system of hybrid biomass power-to-methanol system showed that MILP discovered optimal sizing and dispatch that results in LCOM of 805 EUR /t under the assumption of high biogas availability. This system adopts a configuration consisting of a low-pressure AWE, absorption-based CO2 capture, and steam production from methanol purge gas for internal heat supply [10.1515/psr-2020-0058]. In one energy-community case, MILP has been used to optimize jointly design and operate distributed polygeneration with district heatng and cooling, storage, and energy sharing, showing clear cost-emissions tradeoffs. The result showed two results can come out depending on objective function emphasis of either 24.3% CO2 emission cut at 1.9% cost increase or 31.9% CO2 cut with a 2.22.2% CO2 emissions increase [10.3390/en17133085].

The key strength of MILP is the deterministic model that it offers on the modeled problem. If the system can be expressed with linear relationships, MILP guarantees an optimal solution to that formulation, which makes it highly attractive for sizing and dispatching problems with hard operational constraints []. Other than that, every constraint, feasibility, and infeasibility are explicit and can be traced to how the system is defined, thus it has the benefit of easier interpretability and debugging. Due to the exactness and deterministic model, MILP becomes a great baseline for benchmarking novel methodologies such as RL [].

The main gap of MILP is that optimality is guareanteed only for the approximated linear model, not necessarily the real physical system []. Therefore, in the case of complex polygeneration system that contains nonlinear part-load efficiencies, thermal inertia, degradation, recycle interactions, and stochastic renewables, they often need to be simplified to their linear equivalent or linearized thorugh process such as piecewise linearization []. Another significant drawback is also the computation time that significantly inreases with the number of non-convex definition, decision variables, and long horizon, which is the exact opposite to a trained RL agent that can run in millisecond after being trained, and results in a major limitation for real-time operation [].

The reason why RL becomes interesting to explore is not because MILP is weak at optimization, rather that it is less suitable for fast repeated control when the environment is uncertain, partially observed, and dynamically changing [10.3390/en18174489, 10.48550/arxiv.2305.05484, 10.3390/electronics13081459]. In biomass power-to-methanol systems, this would matter when biomass quality, renewable power input, hydrogen availability, market signals, and flexible downstream deman vary over time, forcing frequent re-optimization under nonlinear couplings. In this nische, RL literature positions itself as a complementary method rather than a direct replacement for MILP, and MILP remains a robust baseline to validate the results of this RL approach [10.48550/arxiv.2305.05484].

## State-of-the-art RL in Optimizing Energy System and Industrial Processes

The adoption of RL to optimize an energy system or industrial process has been an interest of a lot of researchers due to their compelling advantages, particularly due to their inherent flexibility and adaptability [19]. By directly interacting with a system it is thrown into, the RL agent can derive optimal strategies through a series of Markov Decision Process (MDP). This process involves a dynamic evolution of the states, actions, transitional probabilities, and reward at each time step, where RL agent selects and executes action, at a current state, and receiving a reward from its action.

- Diagram of MDP process

The main objective of an agent is to maximize the discounted sum of reward from time t to infinity, denoted by Rt , which is expressed as follows

- Equation of sum of reward

The agent navigates its action based on a policy π, which directs agents what to do given the states of the environment. Each policy is associated with a state-value function Vπ(s), which predicts the result for state s, when following policy π:

- Equation of state-value function

The optimal value of state s,  V*(s) is the defined as Vπ(s) to map the maximum value ofver all possible policies:

- Optimal value of state s equation

The other related variable is a action-value function Qπ(s,a), which gives the expected return when taking action a in state s and following policy π:

- Qπ(s,a) equation

The optimal Q-value of state-action pair, Q*(s,a), is the maximum Q-value over all possible policies:

- Equation of optimal Q-value of state-aciton pair

In short, an optimal policy π*(s) is a policy whose state-value function is equal to the optimal value of state.

- Diagram of these variables as an MDP process (Find available literature online and recreate)

Modelling an accurate environment’s dynamics through MDPs is emphasized by many researchers in the field to obtain a robust framework and good result [20]. Adopting this process enables RL agents to adapt to complex and dynamic environments effectively, which is ideal for optimizing the operation of a polygeneration plant.

- Some examples of RL agent used in optimizing energy systems

There are different types of RL algorithms to be chosen for complex energy system optimization and control. The type of RL algorithms are usually divided into three parts: value-based RL, policy-based RL, and actor-critic that sits in between them. The different between these different types lie in the learning methodology for the RL agent. Value-based RL learns a value function, usually a Q(s,a) estimate, and then chooses an action that returns best estimated value. Some example of value.based RL are Q-learning and Deep Q Network (DQN) [21]. A study argues that value-function methods can run into convergence trouble, especially when the system is in continuous, multivariate settings. This algorithm excels in controlling and optimizing discrete choices but not continuous ones [22].

On the other hand, Policy-based RL learn the Policy directly, which means it parametrizes the action rule itself and pinpoints the optimal policy with methods like policy gradients and PPO [23]. There are many literature on RL that simply applies a value-based approach, however, since energy problem are often continuous, high-dimensional, and constrained, policy-based approach are usually preferred [24].

Actor-critics methods are a mixed between the two: the actor learns the policy, while the critic learns the estimated value to make training more stable. In the literature, earlier studies still adopted Q-learning, but newer research starts to implement actor-critic and other deep RL variants for complex control tasks [25]. This actor-critic method includes PPO, SAC, A3C, TD3, and DDPG. Many studies have have shown the application of this algorithm method into different kinds of energy systems. To be continued…

## RL vs MILP for Different Applications

In recent years, there have been multiple attempts in the literature to benchmark the results of RL and MILP for various energy system problems. Vetter et al. [26] analyzed the implementation of deep reinforcement learning framework using Deep Q-Network (DQN) algorithm to optimize the control of heat pump and heating element operation in a Grid-PV-Thermal Energy Storage (TES) system for an energy community. In this study, both frameworks are used to only optimize the dispatch strategy of the energy system in a fixed sizing environment. Furthermore, the RL agent’s action is only limited to controlling the SOC of the thermal energy storage, while observing its impact on the environment or state it is able to see. On the other hand, the MILP framework decides the flow of grid power and heating element at each time step. The result of this research shows that both frameworks successfully produce yearly cost savings compared to the reference scenario with MILP having greater cost savings on 10-6% reduction compared to a 8.79% RL. Aside from the cost result, RL solution leads in terms of other metrics such as CO2 emission reduction, self-consumption ratio, and self-sufficiency ratio, without it explicitly optimizing it. This is due to the fact that RL’s decision in using more of PV, which increases SFR which further reduces CO2 emissions.

Other than benchmarking the two frameworks for operational dispatch, some researchers also conducted a comparative analysis on RL and MILP for optimizing both the design and control decisions. Cauz et al. [27] used Battery-PV systems as their subject of analysis in order to test and optimize the performance of RL and MILP. The RL algorithm used here is the Direct Environment and Policy Search (DEPS). The design problem here lies in sizing the battery and PV power whereas the control problem lies in dispatching battery and grid operations. The comparison is done in two stages: first stage only involves one week of operation and the second stage tests both frameworks performance in one year case study. For the first stage, the authors discovered that under a fixed sizing scenario, both frameworks manage to converge to global optimum. However, while introducing the design decision problem, the RL at first managed to produce a better result in terms of cost, however, the authors later found that this is due to the modelling limitations that the RL excludes which is the battery SOC state at the end of the simulation that should be the same as the initial SOC state. With this assumption being included in the calculation, the cost result of RL drops and MILP prevails to become cheaper. This shows how important the modelling phase and identical assumption are for a fair comparison. Interestingly, the RL approach manages to converge to a near optimal solution with a design variable that is significantly different from the MILP: 62 kWh and 41 kWp for RL and 40 kWh and 103 kWp for MILP for battery and PV respectively, with the RL layout having 145.65% cost of MILP after considering the additional SOC calculations.

In the second stage of this study, RL approach failed to converge to the same cost value of MILP under the fixed sizing scenario, with it being 12% more expensive. Under the joint operational and sizing decision optimization scenario, RL ends up having 22% more cost than MILP, while the designs decision are significantly smaller than MILP (44 kWh & 57 kWp RL and 95 kWh & 81 kWp for MILP for battery and PV respectively). According to the authors of the study, this poorer performance of RL could be due to a lot of reasons, two of the notable reasons include hyperparameter tuning and outdated chosen RL algorithm that was developed in 1992.

## Hybrid Bayesian Optimization and MILP or RL as Bilevel Formulation

# Methodology

## Hybrid Biomass Power-to-Methanol System Model

- Energy system definition

- Technical Parameters definition

- Economical Parameters definition

## Two-steps Outerloop & Innerloop Strategy

## Level 2 Optimization Architecture (MILP)

- Component level non-linear modelling logic

- Operation logic

  - What tool is used to model the MILP problem? Oemof.solph

  - What solver? CBC

- Objective function definition

## Level 3 Optimization Architecture (RL)

- Component level linear modelling logic

- Chosen RL Algorithm

- Operation logic (MDP)

  - How does BO and RL work together

- Objective function definition (Reward function)

## Data

## Comparison Stages & Scenario Definition

- This comparison will be done in two steps:

  - Outerloop comparison

    - The innerloop operational decision is fixed as an oemof problem,

    - for the outerloop (the process to select what layout to feed into the inner loop), different tools such as oemof investment block (IB), Bayesian Optimization (BO), and Design of Experiment (DOE) are compared

    - This outerloop comparison is to pinpoint which approach results in the best layout both under linear and non-linear economic of scale assumption.

    - Components to optimize: Electricity Supply & Power to X Optimization

  - Innerloop comparison:

    - two innerloop operational dispatch optimization strategies: RL agent living in a physics-based model of the plant vs oemof linear model of the plant are compared.

    - Stage 1: Fixed Outerloop: Linear Model with RL

      - How close can RL can optimize a linear model compared to level 2?

    - Stage 2: Fixed Outerloop: Non-Linear Model with RL

      - What happens to the optimized linear layout when it is injected into real physics-based model?

      - What happens when level 2 framework layout result is fed into level 3 model? Does it remain feasible? And is it still a good solution?

      - What is the difference between the result of simplified linear model vs complex nonlinear model?

    - Stage 3: Both outerloop & Innerloop BO RL Nonlinear vs BO Oemof Linear pathway

      - How different is the optimal results of a simplified linear model and accurate complex physics-based model?

- There are four case studies for each step to be analyzed (For innerloop 3 comparison):

  - Weather Variability:

    - Stable solar and wind input

    - Extreme weather volatility

  - Market Conditions:

    - Cheap batteries

    - Cheap hydrogen system (or maybe -50%, -20%, +20%, +50%)

  - System Design & Flexibility:

    - Heavy fixed methanol reactor load

    - Heavy fixed electrolyzer load

- For each of the steps, stages, and case studies, there are some predetermined KPIs to be examined:

  - Techno-economic level:

    - LCOM

    - CAPEX & OPEX

    - Carbon intensity

    - Grid dependency ratio

    - Self-consumption Ratio

    - Autonomy days

  - Framework level:

    - Simulation time

    - BO Distance

  - Scenario:

  - Year:

  - CAPEX and OPEX per component:

  - Conversion Factor per component:

  - Fixed components:

    - MeOH reactor

    - Distillery

    - Storage MeOH

    - Anaerobic Digester

    - ATR

    - O2 Storage: Full load of ATR sizing x3 days

    - Steam generator

  - AUX:

    - RWGS

    - Fuel Boiler

    - AD Gas Separator (Size depends on )

    - Compressor

    - 3 Heat Exchangers

  - Variable components: (IB vs BO vs DOE)

    - CH4 Generator or GT

    - Wind turbine

    - PV

    - BESS

    - Electrolyzer

    - Hydrogen storage

    - Biogas storage

# Results and Discussions

# Conclusions

# References

[1]	S. Stavrev and D. Ginchev, “Reinforcement Learning Techniques in Optimizing Energy Systems,” Apr. 01, 2024, Multidisciplinary Digital Publishing Institute (MDPI). doi: 10.3390/electronics13081459.

[2]	J. Langford, “Efficient Exploration in Reinforcement Learning,” in Encyclopedia of Machine Learning, Boston, MA: Springer US, 2011, pp. 309–311. doi: 10.1007/978-0-387-30164-8_244.

[3]	Y. Liu, J. Yang, L. Chen, T. Guo, and Y. Jiang, “Overview of Reinforcement Learning Based on Value and Policy,” in 2020 Chinese Control And Decision Conference (CCDC), IEEE, Aug. 2020, pp. 598–603. doi: 10.1109/CCDC49329.2020.9164615.

[4]	R. Lincoln, S. Galloway, B. Stephen, and G. Burt, “Comparing Policy Gradient and Value Function Based Reinforcement Learning Methods in Simulated Electrical Power Trade,” IEEE Transactions on Power Systems, vol. 27, no. 1, pp. 373–380, Feb. 2012, doi: 10.1109/TPWRS.2011.2166091.

[5]	Y. Gao, “Policy Gradient Methods in Deep Reinforcement Learning,” Applied and Computational Engineering, vol. 158, no. 1, pp. 27–34, May 2025, doi: 10.54254/2755-2721/2025.TJ23321.

[6]	D. Cao et al., “Reinforcement Learning and Its Applications in Modern Power and Energy Systems: A Review,” Journal of Modern Power Systems and Clean Energy, vol. 8, no. 6, pp. 1029–1042, 2020, doi: 10.35833/MPCE.2020.000552.

[7]	H. Shengren, E. M. Salazar, P. P. Vergara, and P. Palensky, “Performance Comparison of Deep RL Algorithms for Energy Systems Optimal Scheduling,” in 2022 IEEE PES Innovative Smart Grid Technologies Conference Europe (ISGT-Europe), IEEE, Oct. 2022, pp. 1–6. doi: 10.1109/ISGT-Europe54678.2022.9960642.

[8]	V. Vetter, P. Wohlgenannt, P. Kepplinger, and E. Eder, “Deep Reinforcement Learning Approaches the MILP Optimum of a Multi-Energy Optimization in Energy Communities,” Energies (Basel)., vol. 18, no. 17, p. 4489, Aug. 2025, doi: 10.3390/en18174489.

[9]	M. Cauz, A. Bolland, B. Miftari, L. Perret, C. Ballif, and N. Wyrsch, “Reinforcement Learning for Joint Design and Control of Battery-PV Systems,” Jul. 2023, [Online]. Available: http://arxiv.org/abs/2307.04244